This Lift Plan is the procedural document; the math lives in Critical_Lift_Calculator.xlsx. Together they form a complete Critical Lift Plan compliant with 29 CFR 1926 Subpart CC (Cranes & Derricks). Routine production lifts use Section 4 (Standard Lift Procedure) only; Critical lifts trigger Section 5+ and require the calculator + a written permit (Section 11) signed by the Lift Director before the lift begins.

Section 1 — Roles & Qualifications (1926.1427-1432)

Section 2 — Equipment & Inspections (1926.1412)

Section 3 — Rigging Standards (1926.251 / ASME B30.9)

Section 4 — Standard Lift Procedure

Section 5 — Critical Lift Criteria & Required Steps

Section 6 — Tandem Lift Addendum

Section 7 — Personnel Platform ("Man Basket") Addendum (1926.1431)

Section 8 — Energized Power Line Clearance (1926.1408)

Section 9 — Ground Conditions & Bearing Pressure (1926.1402)

Section 10 — Stop-Work Triggers

Section 11 — Critical Lift Permit (Printable)

| Role | Qualification | Designation |
| --- | --- | --- |
| Operator (1926.1427) | NCCCO, state-licensed, OEM-certified, or audited employer program — type-specific. Medical fitness current within 12 months. | Per shift, on the crane. |
| Lift Director (1926.1432(b)) | Person designated in writing by the employer; experienced in crane operations and rigging; responsible for the planning and execution of the lift. | Per lift. May be foreman or site super for routine lifts; senior PM or crane vendor lead for critical. |
| Signal Person (1926.1428) | Qualified by third-party qualifier OR by employer's qualified evaluator — knowledge of standard signals + crane dynamics. | Required whenever the lift site is not in full direct view of the operator. |
| Rigger (1926.251 / ASME B30.9) | Trained in sling selection, capacity calculation, hitch geometry, sling-inspection criteria. | Selects and applies the rigging; verifies before lift. |
| Spotter — energized lines (1926.1408(c)) | Dedicated to watching crane proximity to power lines. Separate from signal person. | Required when working within Table A clearance plus 20 ft. |
| Tagline Handler(s) | Trained in tagline use; minimum two tag lines on any load > 2,000 lb. | One per tag line. |

| Designated Lift Director for this project: | [Name + signed designation letter on file] |
| --- | --- |
| Designated Operator(s): | [Name(s) — NCCCO/state # — expiration] |
| Designated Signal Person(s): | [Name(s)] |
| Rigger: | [Name + ASME B30.9 trained] |

| Inspection | Frequency | Reference | Last performed |
| --- | --- | --- | --- |
| Daily / shift | Each shift before use | 1926.1412(d) | Daily Field Report |
| Monthly | Every month | 1926.1412(e) | [Date] |
| Annual / comprehensive | Every 12 months | 1926.1412(f) | [Date / Certified Inspector] |
| Modified, repaired, adjusted | After any modification | 1926.1412(g) | As-occurs |
| Post-assembly | After assembly before use | 1926.1412(c) | As-occurs |
| Wire rope | Per ASME B30.5 + 1926.1413 | 1926.1413 | Daily + monthly |

| Make / Model: | [Make / Model] |
| --- | --- |
| Capacity (boom configuration): | [Tons] |
| Boom length: | [ft] |
| Jib / fly configuration: | [Length + offset] |
| Counterweight: | [lbs] |
| Reeving: | [# parts of line] |
| Hook block weight: | [lbs] |

Sling rated capacity at the actual hitch (vertical / choker / basket) must equal or exceed the share of the load on that sling, with appropriate de-rating for sling angle.

Sling angle below 60° from horizontal de-rates capacity sharply — sling load = (load × share) ÷ sin(angle). At 30° the sling sees 2× the vertical share.

Choker hitch capacity is 75% of vertical for synthetic; 80% for wire rope.

Wire-rope clips installed per 1926.251(c)(5) Table H-20 — number, spacing, torque.

Synthetic slings inspected for cuts, snags, melted fibers, broken stitching, missing tag, sun damage.

Sling tags must be legible and intact. Untagged slings are removed from service.

Shackles: bolt or screw-pin, rated capacity stamped, no field substitutions, no painted-over capacity stamps.

Hooks: rated capacity, equipped with safety latch (closed when in use), no spreading or wear >10% of original section.

Eyebolts: shoulder type only for angular loads; torqued to manufacturer spec; thread engagement minimum 1× diameter.

Spreader bars: rated and certified; rigging hooks at correct geometry for the pick.

Minimum two tag lines for any load > 2,000 lb. Tag lines installed and tested before the load leaves the ground. Tag line operators positioned outside the exclusion zone, in clear sight of the signal person.

☐ Pre-shift crane inspection completed and signed off.

☐ Pre-shift rigging inspection — slings, shackles, hooks, tag lines — completed.

☐ Load chart referenced; Lift Director confirms pick weight is within capacity at the lift radius (apply de-rating for outrigger config, on-rubber if not outrigged, side-load, etc.).

☐ Ground conditions confirmed adequate (firm, level, blocked under outriggers; mat / pad if soft).

☐ Swing path surveyed — overhead obstructions, energized lines, adjacent structures.

☐ Energized power line clearance verified per §1926.1408 Table A; spotter assigned if within trigger distance.

☐ Exclusion zone established below the load and along the swing path; signage / barriers as needed.

☐ Tag lines attached (minimum two for loads > 2,000 lb).

☐ Communication channel confirmed (radio frequency + verbal handshake with operator).

☐ Lift Director's GO command before lift.

☐ Initial lift — load raised just off support, paused, all rigging verified visually + audibly (no creak / pop).

☐ Slow swing to placement point.

☐ Connectors at receiving position with PPE + fall protection compliant.

☐ Load placed; minimum two bolts installed wrench-tight per §1926.756(a)(1) before load release.

☐ Load released from rigging; tag lines retrieved after bolts are in.

☐ Operator returns block to neutral / safe position.

| Pick weight ≥ 75% of crane rated capacity at the lift radius | ☐ Trigger |
| --- | --- |
| Multi-crane (tandem) lift | ☐ Trigger — see Section 6 |
| Personnel suspended on the load ("man basket") | ☐ Trigger — see Section 7 |
| Lift over occupied structure (occupied at time of lift OR cannot be evacuated) | ☐ Trigger — EOR review required |
| Lift over public way / energized lines without engineered clearance | ☐ Trigger |
| Lift requires crane to be on-rubber (no outriggers) | ☐ Trigger |
| Lift requires reaching over the rear of the crane | ☐ Trigger |
| Element weight ≥ 40,000 lb (firm-specific threshold; verify against company policy) | ☐ Trigger |
| Lift uses non-standard rigging (custom spreader, unusual geometry, unbalanced load) | ☐ Trigger |
| Other principal-designated critical lift | ☐ Trigger |

Engineering review. Critical Lift Calculator XLSX completed; calculator outputs reviewed by Lift Director + a qualified person (in-house engineer or crane vendor lead).

EOR notification (when over occupied structure, atypical rigging, or where the prime contract specifies).

Pre-lift huddle. Every participant in the lift attends: operator, signal person, rigger, tag-line handlers, connectors, Lift Director, safety officer, GC rep. Walk the procedure, confirm communication, confirm exclusion zone.

Written Critical Lift Permit. Section 11 form completed and signed. Posted at the crane during the lift.

Weather window. Wind sustained ≤ 20 mph, gust ≤ 30 mph (more conservative than routine). No precipitation that affects rigging visibility.

Trial lift. For personnel-platform lifts ONLY (1926.1431) — required by regulation; for other critical lifts, an inch-up confirmation is sufficient.

Stop-work authority. Any participant may stop the lift at any moment for any reason; permit is voided and re-issued only after the issue is corrected.

Post-lift review. Within 24 hours, the Lift Director documents any deviation from plan and what was learned, for future critical-lift planning.

| Crane 1 (lead): | [Make/Model — capacity — operator] |
| --- | --- |
| Crane 2 (assist): | [Make/Model — capacity — operator] |
| Capacity check — each crane: | ☐ Calculator confirms each crane carries ≤ 75% of capacity at its respective radius |
| Operators training together: | ☐ Joint pre-lift walk-through completed |
| Single Lift Director with override authority: | [Name] |
| Communication channel: | [Single radio channel — all operators + signal person] |
| Sequence of movements: | [Documented — pick / level / swing / set] |
| EOR review: | ☐ Reviewed — date[Date] |

Use only when conventional means are more hazardous. If a scissor lift, boom lift, scaffold, or stair-built reach is feasible, that method is required. The personnel-platform option is a last resort.

| Justification for platform use: | [Why conventional means are more hazardous / not possible] |
| --- | --- |
| Platform certification: | ☐ Engineered and certified to ASME B30.23 + 1926.1431(e); rated load capacity stamped |
| Trial lift completed (1926.1431(k)(1)): | ☐ Yes — date/time[Date] |
| Pre-lift meeting (1926.1431(k)(2)): | ☐ Conducted with all participants |
| Crane load limit (1926.1431(c)): | Hoisted load + platform + occupants + tools ≤ 50% of crane's rated capacity at radius. |
| Occupant PFAS (1926.1431(k)(10)): | ☐ Each occupant uses a body harness or restraint belt attached to platform or crane lower load block (NOT to the crane's load line) |
| Operator stays at controls (1926.1431(k)(9)): | ☐ Operator remains at controls the entire time the platform is occupied |
| Free fall prevention: | ☐ Crane equipped with anti-two-block, load-limiter, brakes; no free fall in raise or lower |

| Line Voltage (phase to phase) | Minimum Clearance — Operating | Minimum Clearance — Traveling |
| --- | --- | --- |
| 0 to 50 kV | 10 ft (3.05 m) | 4 ft (1.22 m) |
| Over 50 kV to 200 kV | 15 ft (4.6 m) | 10 ft (3.05 m) |
| Over 200 kV to 350 kV | 20 ft (6.1 m) | 15 ft (4.6 m) |
| Over 350 kV to 500 kV | 25 ft (7.62 m) | 20 ft (6.1 m) |
| Over 500 kV to 750 kV | 35 ft (10.67 m) | 30 ft (9.14 m) |
| Over 750 kV to 1000 kV | 45 ft (13.72 m) | 40 ft (12.19 m) |
| Over 1000 kV | (Established by utility / engineer) | (Established by utility / engineer) |

De-energize and ground the line — confirmed in writing by utility.

OR maintain the Table A clearance, with: (a) planning meeting before the lift, (b) dedicated spotter, (c) visible warning device (banner / cone / proximity alarm), (d) operator orientation, (e) elevated warning line (tape / barrier) where applicable.

OR insulating link/barrier between the line and crane (engineered, not improvised).

| Lines on this project: | [Voltage(s) + utility company + position relative to crane swing] |
| --- | --- |
| Method selected: | ☐ De-energize ☐ Maintain Table A + spotter ☐ Insulating barrier |
| Spotter assigned: | [Name] |
| Utility 24-hour contact: | [Name + phone] |

| Ground bearing capacity at crane positions (soils report): | [psf] |
| --- | --- |
| Outrigger pad / mat dimensions: | [L × W × thickness — material] |
| Ground bearing pressure at max lift (calculated): | [psf — see Critical Lift Calculator] |
| Slope at crane position: | [% — crane manufacturer's max slope] |
| Underground utilities / voids in crane footprint: | ☐ None identified by 811 ticket ☐ Variance — see EOR memo |
| Below-grade structures (vaults, parking levels): | ☐ Verified by EOR — load distribution OK ☐ N/A |
| Rain / freeze-thaw effect on ground: | ☐ Site Super reassesses after weather events |

Any one of the following triggers an immediate stop. The Lift Director (or any participant with stop-work authority) calls "STOP" on the radio channel. The operator brings the load to a safe position. Resume only after the trigger condition is resolved.

| Trigger | Required Response |
| --- | --- |
| Wind sustained >25 mph (routine) / >20 mph (critical) | Set the load and ground the crane (boom down / parking position). |
| Wind gust >35 mph (routine) / >30 mph (critical) | Set the load and ground the crane. |
| Lightning within 6 miles | Personnel off the crane; load set; crane grounded. Resume 30 min after last strike. |
| Heavy rain affecting visibility | Set the load; reassess after rain abates. |
| Power-line encroachment alarm or visible approach | Stop motion; back off to clearance; reassess. |
| Crane functional fault (LMI, anti-two-block, brake, hoist) | Set the load on closest stable support; lock out the crane. |
| Rigging anomaly (audible pop, visible deformation, sling shift) | Set the load; remove damaged rigging from service. |
| Personnel in exclusion zone | Hold load; clear zone; resume. |
| Ground settlement / outrigger sinking | Set the load; reassess ground; re-pad. |
| Communication failure (radio or signal) | Hold load until communication restored. |
| Operator unable to continue safely | Set the load; transition to backup operator; reassess. |

Print one copy per critical lift. Complete during pre-lift huddle. Sign at bottom. Post at crane during the lift. File in project record after.

| Permit No.: | [Seq #] | Date / Time: | [Date / HH:MM] |
| --- | --- | --- | --- |
| Project: | [Project Name] |  |  |
| Lift Description: | [e.g., Set transfer girder TG-3 at Gridline G/4, El. 38'-0", weight 58,200 lb] |  |  |
| Crane(s): | [Make/Model + capacity] |  |  |
| Lift radius / capacity: | [ft / lb] | % capacity: | [%] |
| Critical trigger(s) (Section 5): | [Which trigger(s)] |  |  |

☐ Calculator XLSX completed and reviewed.

☐ Pre-lift huddle held with operator, signal person, rigger, connectors, tag-line handlers, safety officer.

☐ Crane inspected this shift; no defects.

☐ Rigging inspected; sling angle ≥ 60°; capacity adequate for share + angle de-rate.

☐ Ground bearing pressure within soils-report capacity (with factor of safety).

☐ Energized-line clearance verified per Section 8.

☐ Exclusion zone established; barriers / signage in place.

☐ Tag lines attached and tested.

☐ Communication channel confirmed; backup channel identified.

☐ Weather check — wind ≤ thresholds; no precipitation issue; no lightning < 6 mi.

☐ Receiving connectors in position with PFAS.

☐ EOR review (if over occupied structure): date [Date].

☐ Stop-work authority confirmed; every participant briefed.

| Lift outcome: | ☐ As planned ☐ Deviations:[Describe] |
| --- | --- |
| Stop-work events: | ☐ None ☐ Documented separately |
| Lessons learned (for future planning): | [Notes] |

Document version: 2026-05-17 (v1.0) — ContrPro Steel Erection trade pack. Procedures parallel 29 CFR 1926 Subpart CC, ASME B30.5 / B30.9 / B30.10 / B30.20 / B30.23 / B30.26, and AISC Code of Standard Practice without reproducing copyrighted material. Federal regulations current as of January 2026; verify against latest CFR + ASME publications.