Print this checklist for the pre-construction coordination meeting. Walk every line item
        with the GC superintendent, the EOR's plumbing engineer, and the AHJ plumbing inspector
        where available. Anything unresolved becomes an action item with an owner and a date.
        Sign at Section 14. File the signed copy with the project's contract documents.

Project Identification & Attendance Roster

Contract Documents & Scope Confirmation

Permit & AHJ Confirmation

Drawing & Spec Review (Plumbing + Coordination)

Submittal Schedule

Long-Lead Material Tracking

Site Logistics & Mobilization

Multi-Trade Coordination Plan

Inspection Sequence & AHJ Workflow

Safety Program Confirmation (links to Site-Specific Plumbing Plan)

Insurance, Bonds & Licenses

Schedule, Manpower Forecast & First-Day Plan

Action Items

Signatures

| Project Name + Number | [Project Name / GC project #] |
| --- | --- |
| Project Address | [Address] |
| Plumbing Contract Value | [$] |
| Plumbing Contract Notice-to-Proceed | [Date] |
| Substantial Completion Target | [Date] |

| Role | Name | Company | Phone / Email | Initial |
| --- | --- | --- | --- | --- |
| GC Project Manager | [fill] |  |  |  |
| GC Superintendent | [fill] |  |  |  |
| GC Safety Officer | [fill] |  |  |  |
| Plumbing Project Mgr | [fill] |  |  |  |
| Plumbing Superintendent | [fill] |  |  |  |
| Plumbing Foreman | [fill] |  |  |  |
| Master Plumber of Record | [fill] |  |  |  |
| Plumbing Engineer (EOR) | [fill] |  |  |  |
| AHJ Plumbing Inspector | [often async — note last contact] |  |  |  |
| Mechanical Sub Rep | [fill] |  |  |  |
| Electrical Sub Rep | [fill] |  |  |  |
| Fire-Protection Sub Rep | [fill] |  |  |  |
| Concrete Sub Rep | [fill] |  |  |  |
| Owner's Rep | [if attending] |  |  |  |

Signed plumbing subcontract on file (or owner-direct PO).

All exhibits attached (drawings, specs, schedule, insurance reqs, scope statement, exclusions).

Scope boundaries with adjacent trades documented in writing (HVAC, FP, EC, site utility).

Inclusions / exclusions list signed by both parties.

Change-order process documented — who can authorize? What's the dollar threshold? Lead time required?

Pay-application schedule — submission day, retention %, lien-waiver type (conditional/unconditional).

Liquidated damages — daily rate? Curing process?

Warranty period documented — start date trigger, scope of warranty work.

| Plumbing Permit Issued? | Y / N — permit # + date |
| --- | --- |
| Gas Permit Issued? | Y / N / not required |
| Cross-Connection / Backflow Permit | Y / N / by water purveyor |
| Adopted Plumbing Code Edition | IPC 2024 / UPC 2024 / amendment |
| Inspection Request System | Online portal / phone / email + portal URL |
| Inspector Lead Time | [hours / business days] |
| Re-Inspection Fee | [$ per re-trip] |
| Tap-In Coordination | Water purveyor coordinator + phone |
| Sewer Connection Auth | Sewer authority coordinator + phone |

Current revision set on hand at GC trailer + plumbing field trailer.

All RFIs to date logged + dispositioned.

P-0.0 / 1.0 series plumbing plans reviewed against P-2.0 / 3.0 details.

Fixture schedule + cut sheets cross-referenced against specs (matched manufacturers).

Riser diagrams checked for vertical clearance through structure.

Cross-connection control table identifies device + hazard tier for each fixture / system.

Sleeve + opening schedule reconciled with structural / architectural.

Above-ceiling overlay (BIM or marked-up 2D) reviewed — clashes flagged.

Below-slab utility overlay reviewed — site utility congestion, depth conflicts.

Wall penetrations through rated assemblies — UL listing identified per location.

Roof penetrations — coordinated with roofing manufacturer mock-up.

Trapeze hanger locations — structural attachment approved by EOR.

| Submittal Item | Spec Section | Submit By | Approval Needed By | Status |
| --- | --- | --- | --- | --- |
| Pipe + fittings — DWV | 22 13 16 | [fill] |  |  |
| Pipe + fittings — domestic water | 22 11 13 | [fill] |  |  |
| Pipe insulation | 22 07 19 | [fill] |  |  |
| Valves — general duty | 22 05 23 | [fill] |  |  |
| Backflow preventers (RPZ / DCV) | 22 11 19 | [fill] |  |  |
| Plumbing fixtures + trim | 22 42 | [fill] |  |  |
| Water heaters / boilers | 22 33-35 | [fill] |  |  |
| Grease interceptor | 22 13 23 | [fill] |  |  |
| Sewage ejector pumps | 22 13 29 | [fill] |  |  |
| Firestop assemblies (per UL system) | 22 05 19 | [fill] |  |  |
| Hangers + supports | 22 05 29 | [fill] |  |  |
| Disinfection procedure (AWWA C651) | 22 08 | [fill] |  |  |

Track items with lead times longer than 4 weeks. Stage purchase orders to align with
    when materials are actually needed on site — too early eats cash + storage, too late
    delays inspection.

| Item | Vendor | PO Date | Lead Time | Needed-On-Site Date | Status |
| --- | --- | --- | --- | --- | --- |
| Booster pump system | [fill] |  |  |  |  |
| Custom-fabricated waste pump skid | [fill] |  |  |  |  |
| Specialty backflow (3" RPZ +) | [fill] |  |  |  |  |
| Custom-color fixtures | [fill] |  |  |  |  |
| Medical-gas equipment (if applicable) | [fill] |  |  |  |  |

Site access (gate, entry route, parking, hours of operation).

Lay-down / storage area assigned for plumbing materials.

Temporary power available for tools — locations + amperage.

Temporary water + sanitary identified.

Hoisting / crane / forklift coordination — schedule + responsible party.

Dumpster / waste-disposal location.

Tool-trailer / gang-box location.

Site security — after-hours access procedure + badge issuance.

Daily check-in / check-out + visitor sign-in procedure.

| Coordination Item | Responsible Trades | Resolution Mechanism |
| --- | --- | --- |
| Above-ceiling rough-in priority sequence | Plumb / Mech / Elec / FP | BIM / coordination drawings / weekly meeting |
| Slab sleeves + cast-in-place embeds | Plumb / Concrete | Sleeve schedule submitted 7-10 days pre-pour |
| Stud-wall framing for fixture rough-in | Plumb / Framing | Rough-in markup on framing layout |
| Underground utility tie-ins | Plumb / Site / EC | Locate ticket + utility survey |
| Fire-protection main + service entry | Plumb / FP | Service-entry RFI + DCV/RPZ confirmation |
| Electrical for water heaters / pumps | Plumb / EC | Final-connection scope-split confirmation |
| Insulation around hot piping | Plumb / Insulation sub | Schedule following final pressure test |
| Firestop installation at penetrations | Plumb / Firestop sub | UL-listed system per penetration type |
| Final connection to owner-furnished equipment | Plumb / Owner | Scope-split confirmation in submittal |

Walk the inspection sequence with the GC schedule. Anchor each inspection to a
    schedule milestone. Re-trip fees and missed pours are the most expensive avoidable
    cost on a plumbing job.

| Inspection | Schedule Milestone | Lead Time to Request | Responsible Caller |
| --- | --- | --- | --- |
| Underground / In-Slab | Pre-pour day | [hours] | [name] |
| Above-Slab Rough-In | Pre-drywall | [hours] | [name] |
| Gas Pressure Test | Pre-appliance set | [hours] | [name] |
| Top-Out / Stack Test | Pre-finish | [hours] | [name] |
| Backflow Initial Test | Pre-service | [hours] | [name] |
| Final Plumbing | Pre-TCO | [hours] | [name] |

Site-Specific Plumbing Plan (SSPP) reviewed + signed.

Confined-space program — entry permit form, atmospheric meter calibration, rescue services identified.

Hot-work permit program — daily permit, fire watch, 30-min cool-down.

Excavation Competent Person identified + Subpart P training current.

Hazcom / GHS — SDS available for chemicals (flux, primer, cement, chlorine).

Lockout / tagout program — written procedures, locks issued, training current.

PPE issued — eye, gloves (cut + chemical), hard hats, hi-viz, hearing where required.

Toolbox-talk schedule — weekly minimum, documented attendance.

First-aid kit location + AED location confirmed.

Heat / cold stress protocol per local climate.

| Workers' Comp Certificate | [Effective dates + carrier + limit] |
| --- | --- |
| General Liability Certificate | [Effective dates + carrier + per-occurrence / aggregate limit] |
| Auto Liability | [Effective dates + limit] |
| Umbrella / Excess | [Effective dates + limit] |
| Pollution Liability (if applicable) | [Effective dates + limit] |
| Performance Bond | [Y/N + surety + limit] |
| Payment Bond | [Y/N + surety + limit] |
| Plumbing Contractor License | [State # + expiry] |
| Master Plumber License | [Name + license # + expiry] |
| Backflow Tester License | [Name + state # + expiry] |
| Medical-Gas Brazer (if applicable) | [Name + ASSE 6010 cert # + expiry] |
| DOT MC# (if pulling trailers) | [#] |

| Phase | Weeks | Foremen | Journeymen | Apprentices | Helpers |
| --- | --- | --- | --- | --- | --- |
| Underground rough | [fill] |  |  |  |  |
| Above-ground rough | [fill] |  |  |  |  |
| Top-out | [fill] |  |  |  |  |
| Trim-out / fixtures | [fill] |  |  |  |  |
| Commissioning / punch | [fill] |  |  |  |  |

Site walk by plumbing superintendent + GC superintendent + safety officer.

Locate ticket on file. Hand-dig within tolerance per 811 markings.

Stake the first underground run from the survey + drawing.

Confirm sleeve schedule + slab embeds with concrete foreman.

Verify pressure-test apparatus on site + calibrated.

Verify protective system (sloping / shield) for the first trench depth.

Daily Field Report — log start activity + crew + weather.

| # | Action Item | Owner | Due Date | Closed? |
| --- | --- | --- | --- | --- |
| 1 | [fill] |  |  |  |
| 2 | [fill] |  |  |  |
| 3 | [fill] |  |  |  |
| 4 | [fill] |  |  |  |
| 5 | [fill] |  |  |  |
| 6 | [fill] |  |  |  |
| 7 | [fill] |  |  |  |
| 8 | [fill] |  |  |  |
| 9 | [fill] |  |  |  |
| 10 | [fill] |  |  |  |

The undersigned have attended the pre-construction coordination meeting and concur
    with the action items above.

Distribute copies to: GC trailer, plumbing field trailer, plumbing office, owner's rep.
        File the signed original in the project closeout binder under "Pre-Construction".